Kiosk Disconnected from the Internet

## ⚠️ Network Connectivity Advisory: Ethernet vs Wi-Fi for Evolt Kiosks

### Recommended Network Configuration

Evolt strongly recommends that all kiosks be connected via Ethernet. While Wi-Fi connectivity is supported, it is provided on a best-effort basis only.

### Key Considerations

- Reliability: Ethernet offers a more stable and consistent connection, which is essential for optimal kiosk performance.

- Troubleshooting: If a kiosk is experiencing connectivity issues while on Wi-Fi, we recommend relocating it temporarily to a location where it can be connected via Ethernet. If the issues are resolved without any configuration changes, the root cause is likely related to Wi-Fi.

### Known Wi-Fi Limitations

- Database Corruption: In some cases, high-latency Wi-Fi environments have caused database corruption within the kiosk, resulting in a “white screen” error.

- Signal Strength vs. Coverage: Even if the site has strong overall internet connectivity, the radio signal strength at the kiosk’s physical location may be weak. This can lead to poor or intermittent connectivity.

  - Solution: Try relocating the kiosk closer to a Wi-Fi access point to improve signal strength and stability.

### Summary

For best performance and to minimize support issues:

- Use Ethernet whenever possible

- Use Wi-Fi only when Ethernet is not feasible, and be aware of its limitations

- Relocate the kiosk for testing if Wi-Fi issues are suspected

- Push back on sites where Wi-Fi is proving to be a barrier to support, until the Kiosk can be relocated to a position where Ethernet is present. If only for a short period for diagnostics to occur

#### Step-by-Step Instructions

- Open Network Settings

  - From the Control Panel, press the Network button.

- Wi-Fi Connection

  - Ensure Wi-Fi is turned ON.

  - Confirm that the New Connection button is visible under Wi-Fi.

  - Check that the Wi-Fi dongle is securely plugged into one of the four USB ports located under the screen at the back of the unit.

  - Press New Connection, select the desired network, and enter the password.

  - Once connected, the status should show 100 (connected) and save the profile under Quick Connect.

- Ethernet Connection

  - Plug the Ethernet cable into the Ethernet port at the back of the unit.

  - The scanner should automatically connect without additional configuration.