# I cannot print or access Wi-Fi or scan my NFC card

### Issue

If the Wi-Fi settings do not display a “New Connection” button, despite the Wi-Fi dongle being correctly inserted into the back of the kiosk, the issue may be related to the USB Lock setting.

### Resolution Steps

- Access System Settings

  - From the Control Panel, press the System button.

- Check USB Lock Status

  - If the USB Lock is toggled ON, switch it to OFF.

  - Press Close, then power cycle the kiosk (turn it off and back on).

- Verify Network Settings

  - After the restart:

    - Confirm the USB Lock remains OFF.

    - Check that the “New Connection” button is now visible in the Network Settings.

- USB Device Detection

  - The USB Lock setting also affects detection of other USB-connected devices such as:

    - VTAP Readers use by the eGym solution

    - Printers

  - If these devices are not recognized, ensure the USB Lock is disabled as the first troubleshooting step.

- Remote Restart via Ethernet

  - If the kiosk is connected via Ethernet, you can apply the USB Lock change remotely:

    - Disable the USB Lock.

    - Use TeamViewer to access the command terminal.

    - Run the following command: sudo supervisorctl restart all

    - This command simulates a reboot and applies the new USB Lock setting without needing to power cycle the unit.