Kiosk Will Not Power On

### Issue

If the EVOLT kiosk does not power on, follow these steps to verify the power connection and identify potential hardware issues.

### 🔌 Step 1: Check Power Connection

- Verify Power Cord Placement

  - Ensure the power cord is fully seated into the power pack.

  - Confirm it is plugged into a working outlet or surge protector (use of a surge protector is strongly recommended).

- Check Power Cord Orientation

  - Inspect the connection at the back of the unit:

    - The flat side of the power cord port must be facing upward.

- Confirm Power Switch Position

  - The “ON” position is indicated by the circle with a dot inside (⭘•).

### 💡 Step 2: Check for Internal Power Indicators

- Inspect Vent Lights

  - Look for any lights visible through the vents located under the screen.

- Interpret Light Status

  - Lights visible: Power is reaching the unit, but there may be a fault with the Power Board or Motherboard.

    - Escalate to the EVOLT Support Team to coordinate a hardware repair.

  - No lights visible: Most likely a failure in the power cord or power pack.

    - Escalate to the EVOLT Support Team to arrange a replacement power pack.