Kiosk stuck on a white scrren

### Issue

If the EVOLT scanner displays a white screen for more than one minute and does not respond, follow these steps to attempt recovery.

## ⚠️ Understanding Whitescreen Recovery Mode

When the EVOLT kiosk enters Whitescreen Mode, it is attempting to recover from a perceived system corruption. This corruption may affect either the software or the database.

### What Happens During Recovery

- The kiosk initiates a self-repair process, aiming to restore functionality by reverting to a backup of the last known stable version of its software and database.

- This process is automatic and designed to protect system integrity.

### Recovery Duration

- The time required for recovery can vary depending on the extent of the corruption and the system’s ability to access and restore the backup.

- Users are advised to wait up to 10 minutes before taking further action, as the system may resolve the issue on its own.

### ⏳ Step 1: Wait for Auto-Recovery

- Ask the customer to wait up to 10 minutes.

- The system includes a debugging tool that may automatically recover and restore functionality during this time.

### 🖥️ Step 2: Remote Recovery via TeamViewer

If the white screen persists after 10 minutes:

- Check Remote Access

  - Confirm if the unit is powered on and has an active internet connection.

  - If the TeamViewer ID is documented in the customer’s HubSpot profile, attempt to remote into the unit.

- Run Software Recovery Script

  - If remote access is successful, open the Command Terminal and run the following commands:

- sudo rm -rf linuxkioskinstall.tar.bz2

- sudo wget https://evoltactive.com/active/releases/linuxkioskinstall.tar.bz2

- sudo tar -xvjf linuxkioskinstall.tar.bz2

- cd linuxkioskinstall

- sudo ./install.sh

  - This will install the latest software version and reboot the unit, restoring access to the Control Panel/Home Screen.

- Clean Up Disk Space

  - After reboot, run the following command to remove the installation file:

- sudo rm -rf linuxkioskinstall.tar.bz2

### 💾 Step 3: Manual Recovery (If Remote Access Fails)

If remote access is not possible:

- Send Recovery Files

  - Provide the customer with the necessary files to download and run the recovery process locally.

- USB Re-Image Option

  - Alternatively, the EVOLT Team can send a pre-loaded USB drive to the site.

  - Requirements:

    - A 16GB USB drive

    - A keyboard that can be plugged into the scanner

  - This method allows the customer to re-image the software and restore the unit manually.